# Software Specification – Waveshare 2.9" e-Paper B V3 Driver

Driver version: 1.0
Target platform: MicroPython (tested on ESP32-S3 Zero)
Display: 2.9 inch, 128×296 pixels, black/white/red, SPI interface

## API Overview

| Method | Description |
| --- | --- |
| init() | Initialise the display (call once) |
| show() | Transfer both framebuffers and refresh |
| sleep() | Deep sleep mode |
| clear() | Clear both framebuffers to white |
| fill(color) | Fill entire display with WHITE / BLACK / RED |
| pixel(ch, x, y) | Set a single pixel in BK or RD channel |
| pixel_off(ch, x, y) | Clear a single pixel (set to white) |
| hline(ch, x, y, len) / vline(...) | Horizontal / vertical line |
| line(ch, x0,y0, x1,y1) | Line between two points |
| draw_rect(ch, x, y, w, h) | Rectangle outline |
| fill_rect(ch, x, y, w, h) | Filled rectangle |
| clear_rect(ch, x, y, w, h) | Clear rectangle to white |
| circle(ch, x0, y0, r, filled) | Circle or disk |
| triangle(ch, x0,y0, x1,y1, x2,y2, filled) | Triangle |
| text(ch, txt, x, y, scale=1) | Render text |
| text_center(ch, txt, y, scale=1) | Centered horizontally |
| text_fit(ch, txt, y, max_scale=3) | Automatically choose largest scale |
| text_width(txt, scale) | Width of text in pixels |

## Colour Constants

- EPaper29BV3.WHITE = 0

- EPaper29BV3.BLACK = 1

- EPaper29BV3.RED = 2

Channel selection:

- EPaper29BV3.BK = 0 (black channel)

- EPaper29BV3.RD = 1 (red channel)

## Rotation

Set during construction:

ep = EPaper29BV3(rotation=90)

- Supported: 0, 90, 180, 270

- Logical width and height are automatically swapped for 90° and 270°

Note:
Rotation is applied in software. Internal framebuffer orientation is fixed and does not match logical coordinates directly.

## 1. Class: EPaper29BV3

### Constructor

EPaper29BV3(cs=13, dc=12, rst=5, busy=11, clk=4, mosi=3, miso=16, rotation=0)

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| cs | int | 13 | Chip Select pin |
| dc | int | 12 | Data/Command pin |
| rst | int | 5 | Reset pin |
| busy | int | 11 | Busy pin |
| clk | int | 4 | SPI clock |
| mosi | int | 3 | SPI MOSI |
| miso | int | 16 | SPI MISO (unused) |
| rotation | int | 0 | Display rotation |

### Properties

- width – logical width after rotation (read-only)

- height – logical height after rotation (read-only)

## 2. Initialisation & Power Management

### init() -> None

Initialises the display controller.
Must be called once after power-on.

### show() -> None

Transfers both framebuffers and triggers a full refresh.

- Blocking operation (≈2–3 seconds)

- Waits until BUSY pin goes low

Note:
If the display does not respond, the internal wait may timeout, but no exception is raised.

### sleep() -> None

Puts display into deep sleep.

- Required before cutting power

- Device will not respond until reset or power cycle

### clear() -> None

Clears both framebuffers (sets all pixels to white).
Does not update display — call show() afterwards.

### fill(color: int) -> None

Fills display with a single colour.

| Color | Effect |
| --- | --- |
| WHITE | Clears both channels |
| BLACK | Sets black channel only |
| RED | Sets red channel only |

## 3. Framebuffer & Pixel Model

- Each channel has its own framebuffer

- Size per channel: 128 × 296 / 8 = 4736 bytes

- Bit format:

  - 0 = pixel ON (black/red)

  - 1 = pixel OFF (white)

## 4. Pixel Operations

### pixel(channel, x, y)

Sets a pixel (black or red depending on channel).

- Applies rotation automatically

- Out-of-bounds coordinates are ignored

### pixel_off(channel, x, y)

Clears pixel (sets to white).

## 5. Drawing Primitives

All coordinates are logical (after rotation).

### hline / vline

Draw horizontal or vertical line.

### line

Draw line using Bresenham’s algorithm.

### draw_rect

Draw rectangle outline.

### fill_rect

Draw filled rectangle.

### clear_rect

Clear rectangle to white.

### circle

Draw circle or filled disk.

### triangle

Draw triangle (outline or filled).

## 6. Text Rendering

- Built-in 5×7 pixel font

- ASCII range: 32–122

- Unsupported characters rendered as space

### char(channel, x, y, ch, scale=1) -> int

Draw a single character.

Returns next X position.

### text(channel, txt, x, y, scale=1)

Draw string.

### text_width(txt, scale=1) -> int

Returns pixel width:

len(txt)\\\* (5\\\* scale + scale) - scale

### text_center(channel, txt, y, scale=1)

Draw centered text.

### text_fit(channel, txt, y, max_scale=3) -> int

Finds largest scale that fits screen width.

- Uses margin of 8 pixels

- Returns used scale

## 7. Internal Methods (private)

| Method | Description |
| --- | --- |
| _cmd(c) | Send command |
| _data(d) | Send data |
| _wait(timeout) | Wait for BUSY pin |
| _reset() | Hardware reset |
| _buf(channel) | Get framebuffer |
| _get_char(ch) | Font lookup |

## 8. Constants

| Name | Value | Meaning |
| --- | --- | --- |
| WHITE | 0 | White |
| BLACK | 1 | Black |
| RED | 2 | Red |
| BK | 0 | Black channel |
| RD | 1 | Red channel |
| W | 128 | Physical width |
| H | 296 | Physical height |

## 9. Timing & Dependencies

- SPI speed: 2 MHz

- Refresh time: ~2–3 seconds

- BUSY pin: 1 = busy, 0 = idle

- Uses:

  - machine.Pin

  - machine.SoftSPI

  - time

## 10. Notes

- Display refresh is slow (normal for ePaper)

- Always call sleep() before power-off

- No partial refresh support

## 11. Screenshot Utility (screenshot.py)

Provides a helper function to export the current framebuffer as a BMP image.

### capture(ep, filename="screenshot.bmp") -> bool

Creates a monochrome BMP file from the display framebuffer.

Args:

- ep: instance of EPaper29BV3

- filename: output file name

Notes:

- Uses black channel framebuffer only

- Output resolution is always 128×296 (physical display resolution)

- BMP format: 1-bit monochrome

- Image is vertically flipped to match BMP format

Returns:

- True on success

- False on failure

## 12. Test Program (test_disp.py)

A comprehensive visual test suite demonstrating all driver features.

Features tested:

- Drawing primitives (rectangles, circles, triangles, lines)

- Text rendering (multiple scales, centering)

- Red channel rendering

- Pixel-level operations

- clear_rect functionality

Output:

- Displays full test screen

- Saves screenshot as "test_disp.bmp"

Usage:

import test\_disp  
  
\#\# 13\\. Example

from epaper2in9bv3 import EPaper29BV3
import time

ep = EPaper29BV3(rotation=0)
ep.init()
ep.clear()

ep.circle(ep.BK, 64, 100, 40)
ep.fill_rect(ep.RD, 20, 160, 88, 40)

ep.text_fit(ep.BK, "Hello ePaper", 240)
ep.show()

time.sleep(5)
ep.sleep()

\#\# 14\\. Revision History  
  
|Version|Date|Changes|  
|-|-|-|  
|1.0|2026-04-12|Initial release|  
|1.2|2026-04-17|Specification aligned with implementation|